红外感应计时器

图 1:实物图

1、简要说明：

一、 尺寸：43.7cm *27.0cm *39.5cm   长*宽*高。

二、 主要芯片：博光3-MNK对射式红外感应装置 *1，CD4543 *4, 89C52。

三、 工作电压：数码管8.2v-9.6v   51单片机：5v。

四、 特点：

1、可接电脑同步显示时钟计时器。

2、可屏蔽极短时间内红外多次接收到信号

五、应用：

适用于模型车比赛计时等。

2、传感器说明：

2.1博光3-MNK对射式红外感应装置模块连接说明：

如图所示：

图2

红线接单片机上正极

黑线接单片机上负极

白线接p2.2 作为中断控制，当感应装置中间没有障碍时恒从白线返回一个高电平，否则返回低电平

不可在通电状态下将红（黑）线与白线短接，否则感应器将毁坏

2.2  4英寸共阴LED数码管

Abdskaf\\\\

每一段的电压在8v到11v，小数点的1.5v到3v，小数点极易烧坏需要注意

2.3CD4543BE

详细建议参考百度文库，在这里只介绍其在电路中的搭接，芯片引脚图如下

这里的a-g 对应于数码管的a-g段。芯片连接见原理图。

2.2原理图

图 5：原理图

2.3测试程序

#include <reg52.h>

#include <intrins.h>

#include <stdlib.h>

//unsigned char code   dula[] = {0x3f,0x06,0x5b,0x4f,0x66,0x6d,0x7d,0x07,0x7f,0x6f,0x80};

sbit a = P2^7;

sbit b = P2^6;

unsigned int  numBit[4], iCount;

unsigned int num1 = 0;

unsigned char ch;

unsigned int sig = 0;

void delay(unsigned int n)

{

unsigned int a, b;

for(a = n; a > 0; a--)

for(b = n; b > 0; b--);

}

void init()

{

//interrupt enable

EA = 1;

ET0 = 1;

ES = 1;

//timer 0 for counting, timer 1 for Serial Port

TMOD = 0x21;

TH0 = (65535 - 10000) / 256;

TL0 = (65535 - 10000) % 256;

TH1 = 0xfd;

TL1 = 0xfd;

TR1 = 1;

//	TR0 = 1;

//interrupt0

EX0 = 1;

IT0 = 1;

//reset control

a = 1;

b = 0;

//SCON

SM0 = 0;

SM1 = 1;

REN = 1;

}

void main()

{

init();

while(1)

{

if(sig)

{

sig++;

if(sig == 200)

{

sig = 0;

}

}

if(num1 > 9999)

{

num1 = 0;

//		return ;

}

if(a == b || ch == 'r')//串口发过来’r’使计时器归零

{

num1 = 0;

TR0 = 0;

delay(2);

SBUF = 2;

ch = 0;

}

if(ch == 's')// 串口发过来’s’使计时器终止/运行

{

TR0 = ~TR0;

ch = 0;

}

numBit[0] = num1 / 1000;

numBit[1] = (num1 % 1000) / 100;

numBit[2] = (num1 % 100) / 10;

numBit[3] = (num1 % 10);

P0 = ((numBit[0] << 4)| numBit[1]) & 0x00ff;

P1 = ((numBit[2] << 4)| numBit[3]) & 0x00ff;

}

}

void AnswerTIME0() interrupt 1// using 1

{

TR0 = 0;

TH0 = (65535 - 9216) / 256;

TL0 = (65535 - 9216) % 256;

num1++;

SBUF = 1;//给串口发1给电脑，数字加以1

//		delay(1);

TR0 = 1;

}

void AnswerINT0() interrupt 0// using 1

{

EX0 = 0;

if(P3^2 == 0)

{

delay(3);

if(P3^2 == 0 && sig == 0)

{

sig = 1;

TR0 = ~TR0;

}

}

EX0 = 1;

}

void ser() interrupt 4

{

if(RI == 1)

{

RI = 0;

ch = SBUF;

}

}

另有电脑运行程序：

Main.cpp:

#include <windows.h>

#include <stdio.h>

#include"serailAPI.h"

#include <conio.h>

#define ID_TIMER    1

LRESULT CALLBACK WndProc (HWND, UINT, WPARAM, LPARAM) ;

char ikey, okey;//ikey: input from 51; okey date(8 bit) to 51

int time = 0;

int WINAPI WinMain (HINSTANCE hInstance, HINSTANCE hPrevInstance,

PSTR szCmdLine, int iCmdShow)

{

static TCHAR szAppName[] = TEXT ("Clock") ;

HWND         hwnd;

MSG          msg;

WNDCLASS     wndclass ;

//

wndclass.style         = CS_HREDRAW | CS_VREDRAW ;

wndclass.lpfnWndProc   = WndProc ;

wndclass.cbClsExtra    = 0 ;

wndclass.cbWndExtra    = 0 ;

wndclass.hInstance     = hInstance ;

wndclass.hIcon         = NULL ;

wndclass.hCursor       = LoadCursor (NULL, IDC_ARROW) ;

wndclass.hbrBackground = (HBRUSH) GetStockObject (WHITE_BRUSH) ;

wndclass.lpszMenuName  = NULL ;

wndclass.lpszClassName = szAppName ;

if (!RegisterClass (&wndclass))

{

MessageBox (NULL, TEXT ("Program requires Windows NT!"),

szAppName, MB_ICONERROR) ;

return 0 ;

}

hwnd = CreateWindow (szAppName, TEXT ("MCU Clock"),

WS_OVERLAPPEDWINDOW,

CW_USEDEFAULT, CW_USEDEFAULT,

CW_USEDEFAULT, CW_USEDEFAULT,

NULL, NULL, hInstance, NULL) ;

ShowWindow (hwnd, iCmdShow) ;

UpdateWindow (hwnd) ;

while(TRUE){

if(PeekMessage(&msg,NULL,0,0,PM_REMOVE))//检测是否有消息 有的话处理 否则进行else的内容

{

if(msg.message == WM_QUIT)

break;

TranslateMessage(&msg);

DispatchMessage(&msg);

}

else

{

InvalidateRect(hwnd, NULL, FALSE);                                                                                            //////

SendMessage(hwnd, WM_PAINT,0, 0);

//			Sleep(3);

}

}

return msg.wParam ;

}

LRESULT CALLBACK WndProc (HWND hwnd, UINT message, WPARAM wParam, LPARAM lParam)

{

HDC hdc ;

static HFONT  font;

static int cxClient, cyClient;

char numBit[4];

static HDC hdcBack;

static HBITMAP hBack;

static char sbuf[6];

PAINTSTRUCT ps;

switch (message)

{

case WM_CREATE:

hdc = GetDC(hwnd);

//        SetTimer(hwnd, ID_TIMER, 2, NULL);

hdcBack = CreateCompatibleDC(hdc);

SerailInit();

return 0 ;

case  WM_SIZE:

cxClient = LOWORD(lParam);

cyClient = HIWORD(lParam);

font=CreateFont(cyClient - 10,cxClient/5,0,0,0,0,0,0,GB2312_CHARSET,0,0,0,0,TEXT("方正舒体"));

hBack = CreateCompatibleBitmap(hdcBack, cxClient, cyClient);

SelectObject(hdcBack, hBack);

return 0;

case WM_LBUTTONDOWN:

okey = 's';

return 0;

case WM_RBUTTONDOWN:

okey = 'r';

return 0;

case WM_PAINT:

ikey = ScanSerail();

if(ikey == 1)

time++;

else if(ikey == 2)

time = 0;

numBit[0] = time / 1000;

numBit[1] = (time % 1000) / 100;

numBit[2] = (time % 100) / 10;

numBit[3] = (time % 10);

sprintf(sbuf, "%d%d:%d%d", numBit[0], numBit[1], numBit[2], numBit[3]);

if(okey == 's')

{

WriteChar((BYTE*)"s", 1);

okey = 0;

}

else if(okey == 'r')

{

WriteChar((BYTE*)"r", 1);

okey = 0;

}

hdc = BeginPaint (hwnd, &ps) ;

SelectObject(hdcBack, font);

TextOut(hdcBack, 0, 0, sbuf, strlen(sbuf));

BitBlt(hdc,5,20,cxClient, cyClient, hdcBack, 0, 0, SRCCOPY);

EndPaint (hwnd, &ps) ;

return 0 ;

case WM_DESTROY:

//     KillTimer (hwnd, ID_TIMER) ;

PostQuitMessage (0) ;

return 0 ;

}

return DefWindowProc (hwnd, message, wParam, lParam) ;

}

SeriousAPI.h:

#include <stdio.h>

#include <stdlib.h>

#include <windows.h>

HANDLE hComm;

OVERLAPPED m_ov;

COMSTAT comstat;

DWORD  m_dwCommEvents;

bool openport(char *portname)//打开一个串口

{

hComm = CreateFile(portname,

GENERIC_READ | GENERIC_WRITE,

0,

0,

OPEN_EXISTING,

FILE_FLAG_OVERLAPPED,

0);

if (hComm == INVALID_HANDLE_VALUE)

return FALSE;

else

return true;

}

bool setupdcb(int rate_arg)

{

DCB dcb;

int rate= rate_arg;

memset(&dcb,0,sizeof(dcb));

if(!GetCommState(hComm,&dcb))//获取当前DCB配置

{

return FALSE;

}

/* -------------------------------------------------------------------- */

// set DCB to configure the serial port

dcb.DCBlength      = sizeof(dcb);

/* ---------- Serial Port Config ------- */

dcb.BaudRate       = rate;

dcb.Parity     = NOPARITY;

dcb.fParity    = 0;

dcb.StopBits       = ONESTOPBIT;

dcb.ByteSize       = 8;

dcb.fOutxCtsFlow   = 0;

dcb.fOutxDsrFlow   = 0;

dcb.fDtrControl    = DTR_CONTROL_DISABLE;

dcb.fDsrSensitivity = 0;

dcb.fRtsControl    = RTS_CONTROL_DISABLE;

dcb.fOutX          = 0;

dcb.fInX           = 0;

/* ----------------- misc parameters ----- */

dcb.fErrorChar     = 0;

dcb.fBinary        = 1;

dcb.fNull          = 0;

dcb.fAbortOnError  = 0;

dcb.wReserved      = 0;

dcb.XonLim         = 2;

dcb.XoffLim        = 4;

dcb.XonChar        = 0x13;

dcb.XoffChar       = 0x19;

dcb.EvtChar        = 0;

/* -------------------------------------------------------------------- */

// set DCB

if(!SetCommState(hComm,&dcb))

{

return false;

}

else

return true;

}

bool setuptimeout(DWORD ReadInterval,DWORD ReadTotalMultiplier,DWORD ReadTotalconstant,DWORD WriteTotalMultiplier,DWORD WriteTotalconstant)

{

COMMTIMEOUTS timeouts;

timeouts.ReadIntervalTimeout=ReadInterval;

timeouts.ReadTotalTimeoutConstant=ReadTotalconstant;

timeouts.ReadTotalTimeoutMultiplier=ReadTotalMultiplier;

timeouts.WriteTotalTimeoutConstant=WriteTotalconstant;

timeouts.WriteTotalTimeoutMultiplier=WriteTotalMultiplier;

if(!SetCommTimeouts(hComm, &timeouts))

return false;

else

return true;

}

void ReceiveChar( )

{

BOOL bRead = TRUE;

BOOL bResult = TRUE;

DWORD dwError = 0;

DWORD BytesRead = 0;

char RXBuff;

for (;;)

{

bResult = ClearCommError(hComm, &dwError, &comstat);

if (comstat.cbInQue == 0)

continue;

if (bRead)

{

bResult = ReadFile(hComm,      // Handle to COMM port

&RXBuff,         // RX Buffer Pointer

1,                   // Read one byte

&BytesRead,          // Stores number of bytes read

&m_ov);          // pointer to the m_ov structure

printf("%c",RXBuff);

if (!bResult)

{

switch (dwError = GetLastError())

{

case ERROR_IO_PENDING:

{

bRead = FALSE;

break;

}

default:

{

break;

}

}

}

else

{

bRead = TRUE;

}

} // close if (bRead)

if (!bRead)

{

bRead = TRUE;

bResult = GetOverlappedResult(hComm,  // Handle to COMM port

&m_ov,  // Overlapped structure

&BytesRead,     // Stores number of bytes read

TRUE);          // Wait flag

}

}

}

bool WriteChar(BYTE* m_szWriteBuffer,DWORD m_nToSend)

{

BOOL bWrite = TRUE;

BOOL bResult = TRUE;

DWORD BytesSent = 0;

//	HANDLE m_hWriteEvent;

//	ResetEvent(m_hWriteEvent);

if (bWrite)

{

m_ov.Offset = 0;

m_ov.OffsetHigh = 0;

// Clear buffer

bResult = WriteFile(hComm, // Handle to COMM Port

m_szWriteBuffer, // Pointer to message buffer in calling finction

m_nToSend,      // Length of message to send

&BytesSent,     // Where to store the number of bytes sent

&m_ov );        // Overlapped structure

if (!bResult)

{

DWORD dwError = GetLastError();

switch (dwError)

{

case ERROR_IO_PENDING:

{

// continue to GetOverlappedResults()

BytesSent = 0;

bWrite = FALSE;

break;

}

default:

{

// all other error codes

break;

}

}

}

} // end if(bWrite)

if (!bWrite)

{

bWrite = TRUE;

bResult = GetOverlappedResult(hComm,   // Handle to COMM port

&m_ov,     // Overlapped structure

&BytesSent,    // Stores number of bytes sent

TRUE);         // Wait flag

// deal with the error code

if (!bResult)

{

printf("GetOverlappedResults() in WriteFile()");

}

} // end if (!bWrite)

// Verify that the data size send equals what we tried to send

if (BytesSent != m_nToSend)

{

printf("WARNING: WriteFile() error.. Bytes Sent: %d; Message Length: %d\n", BytesSent, strlen((char*)m_szWriteBuffer));

}

return true;

}

void SerailInit()

{

if(!openport("com2"))////////////////////////////////////////////////////////////////////////

MessageBox(NULL, "open comport failure\n", "error", MB_ICONWARNING);

if(!setupdcb(9600))

MessageBox(NULL, "setupDCB failure\n", "error", MB_ICONWARNING);

if(!setuptimeout(0,0,0,0,0))

MessageBox(NULL, "setuptimeout failure\n", "error", MB_ICONWARNING);

PurgeComm(hComm, PURGE_RXCLEAR | PURGE_TXCLEAR | PURGE_RXABORT | PURGE_TXABORT);

//	WriteChar((BYTE*)"s",1);

}

BYTE  ScanSerail()

{

static BOOL bRead = TRUE;

static BOOL bResult = TRUE;

static DWORD dwError = 0;

static DWORD BytesRead = 0;

static char RXBuff;

bResult = ClearCommError(hComm, &dwError, &comstat);

if (comstat.cbInQue == 0)

//continue;

return 0;

if (bRead)

{

bResult = ReadFile(hComm,      // Handle to COMM port

&RXBuff,         // RX Buffer Pointer

1,                   // Read one byte

&BytesRead,          // Stores number of bytes read

&m_ov);          // pointer to the m_ov structure

//printf("%c",RXBuff);

return RXBuff;

if (!bResult)

{

switch (dwError = GetLastError())

{

case ERROR_IO_PENDING:

{

bRead = FALSE;

break;

}

default:

{

break;

}

}

}

else

{

bRead = TRUE;

}

} // close if (bRead)

if (!bRead)

{

bRead = TRUE;

bResult = GetOverlappedResult(hComm,  // Handle to COMM port

&m_ov,  // Overlapped structure

&BytesRead,     // Stores number of bytes read

TRUE);          // Wait flag

}

//	system("pause");

}

/************************** program end***************************************/

其他:

我们在这里由于驱动问题和CD4543的芯片特性，对数码管是采用静态控制。如果有人要翻做，可以选择动态控制数码管好像更有意思，用一对三极管驱动一段。面白そう！